Safety Risk Assessment Form

| Reference: | RBC Production Generic 004 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date: | 23/02/16 | Assessor(s): | Jonathan Hill |  |  |  | Department/Area: |  |  | Production |  |
| Activity Describe the activity and the ways in which it could pose a risk | Hazards e.g. personal injury, electric shock, people or objects falling from height, crushing | Persons at risk | Risk before control measures |  |  | Controls Describe controls used to reduce risk to a tolerable or preferably acceptable rating | Residual Risk after control measures |  |  | Additional controls? e.g. monitor and review, suggestions for future activities | Owner Person responsible for the controls |
|  |  |  | Severity | Likeli-hood | SxL |  | Severity | Likeli-hood | SxL |  |  |
| Use of electricity | Electrocution, electric shock, burns, flash burns, fire | Operator, persons nearby | 5 | 4 | 20 | NO LIVE WORKING IS PERMITTED UNDER ANY CIRCUMSTANCES ON LOW VOLTAGE (25 to 1000V ac/50V to 1000V DC) SYSTEMS. Wherever possible, final circuits should be protected by a device that automatically shuts off the current in the event of a fault, such as a Residual Current Device with a 30mA 30ms setting. All portable appliances, and the supply to which they will be connected, must be visually inspected before each use and will be subject to a full, recorded inspection every twelve (12) months. Any electrical equipment not suitable for use must not be used, and must be reported to a member of the Technical Instructor team. Wherever possible, battery operated or 'site safe' 110V tools, supplied via a centre earth-tapped transformer should be used. Reel-type extension leads must be fully unwound, and the cable stored appropriately, before use. Multi-way sockets must not be overloaded. Fuses, miniature circuit breakers (MCB) and residual circuit breakers (RCD) must not be bypassed or defeated. | 5 | 1 | 5 |  | Technical Manager, Technical Instructor team, Facilities Manager, operator |

| Risk Scoring Key | Severity |  | Likelihood |  |
| --- | --- | --- | --- | --- |
|  | Fatality | 5 | Certain or imminent | 5 |
|  | Major injury, disabling illness, major damage | 4 | Very likely | 4 |
|  | Lost time injury, illness, damage | 3 | May happen | 3 |
|  | Minor injury, minor damage | 2 | Unlikely | 2 |
|  | Delay only | 1 | Very unlikely | 1 |

| Risk Rating Categories | 10 to 25 | Unacceptable | Do not proceed; seek immediate guidance from the safety team |
| --- | --- | --- | --- |
|  | 6 to 9 | Tolerable | Proceed with caution but seek to reduce risk further if possible |
|  | 1 to 5 | Acceptable | Proceed |

| Order of Control Measures | Most effective | Eliminate | Ask yourself if the activity needs to be carried out |
| --- | --- | --- | --- |
|  |  | Substitute | Ask yourself if the same effect can be achieved with something less risky |
|  |  | Reduce | Ask yourself if you can use less of something, or limit the time etc |
|  |  | Isolate | Make sure that the risk is contained to the smallest possible area |
|  |  | Enclose | Make sure that no-one can get to the hazard |
|  |  | Other Engineering Controls | Emergency stop buttons, automated controls etc |
|  |  | Safe System of Work | Carry out the work according to a specific step by step programme with training |
|  |  | Training/Communication | Safety team can advise |
|  |  | PPE | Use of ear defenders, hard hat, toetectors etc |
|  | Least effective | Discipline and Enforcement | Telling people to be careful |