Safety Risk Assessment Form

| Reference: | RBC Production Generic 008 – Use of hand winch bars (Barn) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date: | 08/02/16 | Assessor(s): | Jonathan Hill |  |  |  | Department/Area: |  |  | Technical |  |
| Activity Describe the activity and the ways in which it could pose a risk | Hazards e.g. personal injury, electric shock, people or objects falling from height, crushing | Persons at risk | Risk before control measures |  |  | Controls Describe controls used to reduce risk to a tolerable or preferably acceptable rating | Residual Risk after control measures |  |  | Additional controls? e.g. monitor and review, suggestions for future activities | Owner Person responsible for the controls |
|  |  |  | Severity | Likeli-hood | SxL |  | Severity | Likeli-hood | SxL |  |  |
| Use of hand winch bars | Bar failing | Persons below | 5 | 4 | 20 | Only operators trained by Technical Instructor team are permitted to raise and lower hand winch bars. Safe working load limit (200kg uniformly distributed) must not be exceeded. | 5 | 1 | 5 |  | Operator |
|  | Damage to bar | Persons below | 4 | 4 | 16 | All impediments to bars being raised or lowered, i.e. multicores, single cables, equipment rigged in the path of the bar, must be lowered or moved clear of the path of travel, before any raising or lowering takes place. Raising or lowering a bar must not exceed the limit of travel of the swivel clew, between the first and second divert pulleys. | 4 | 1 | 4 |  | Operator |

| Risk Scoring Key | Severity |  | Likelihood |  |
| --- | --- | --- | --- | --- |
|  | Fatality | 5 | Certain or imminent | 5 |
|  | Major injury, disabling illness, major damage | 4 | Very likely | 4 |
|  | Lost time injury, illness, damage | 3 | May happen | 3 |
|  | Minor injury, minor damage | 2 | Unlikely | 2 |
|  | Delay only | 1 | Very unlikely | 1 |

| Risk Rating Categories | 10 to 25 | Unacceptable | Do not proceed; seek immediate guidance from the safety team |
| --- | --- | --- | --- |
|  | 6 to 9 | Tolerable | Proceed with caution but seek to reduce risk further if possible |
|  | 1 to 5 | Acceptable | Proceed |

| Order of Control Measures | Most effective | Eliminate | Ask yourself if the activity needs to be carried out |
| --- | --- | --- | --- |
|  |  | Substitute | Ask yourself if the same effect can be achieved with something less risky |
|  |  | Reduce | Ask yourself if you can use less of something, or limit the time etc |
|  |  | Isolate | Make sure that the risk is contained to the smallest possible area |
|  |  | Enclose | Make sure that no-one can get to the hazard |
|  |  | Other Engineering Controls | Emergency stop buttons, automated controls etc |
|  |  | Safe System of Work | Carry out the work according to a specific step by step programme with training |
|  |  | Training/Communication | Safety team can advise |
|  |  | PPE | Use of ear defenders, hard hat, toetectors etc |
|  | Least effective | Discipline and Enforcement | Telling people to be careful |