Safety Risk Assessment Form

| Reference: | RBC Production Generic 010 – Loading and unloading vehicles |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date: | 08/05/2018 | Assessor(s): | Jonathan Hill |  |  |  | Department/Area: |  |  | Technical |  |
| Activity Describe the activity and the ways in which it could pose a risk | Hazards e.g. personal injury, electric shock, people or objects falling from height, crushing | Persons at risk | Risk before control measures |  |  | Controls Describe controls used to reduce risk to a tolerable or preferably acceptable rating | Residual Risk after control measures |  |  | Additional controls? e.g. monitor and review, suggestions for future activities | Owner Person responsible for the controls |
|  |  |  | Severity | Likeli-hood | SxL |  | Severity | Likeli-hood | SxL |  |  |
| Unloading vehicles | Crush injuries, sprains, strains, musculo-skeletal disorders | Operators, persons nearby | 4 | 4 | 16 | The vehicle must have its engine shut off, and the parking brake applied before unloading. If unloading a trailer that has no tractor unit hitched, ensure that the stabilisers are down and secure before anyone board the vehicle. Assess before any unloading for any movement of the load whilst in transit. Operators must be equipped with steel toe-capped footwear to ISO20345:2011, and gloves if appropriate. There must be a sufficient number of operators, both on and off the vehicle to handle items safely. If the unloading activity is taking place on or near a public thoroughfare (even if on private land), operators must wear high-visibility clothing. If the unloading activity is taking place on or near a public thoroughfare (even if on private land), then barriers to demarcate the hazardous area should be erected. As items to be unloaded will have to be handled, all moving and lifting operations must refer to a Manual Handling risk assessment and safe system of work (such as T.I.L.E.) It is recommended that a tail-lift or ramp is used to move loads off the vehicle, especially if the loads have wheels or castors, for example, flight cases or meat-racks. | 4 | 2 | 8 |  |  |
| Loading vehicles | Crush injuries, sprains, strains, musculo-skeletal disorders | Operators, persons nearby | 4 | 4 | 16 | The vehicle must have its engine shut off, and the parking brake applied before loading. If loading a trailer that has no tractor unit hitched, ensure that the stabilisers are down and secure before anyone board the vehicle. Assess before any loading the suitability of the vehicle for the intended load. It is the driver’s responsibility to adequately secure the load with suitable lashings or load-bars. The vehicle operator has a duty to ensure that their vehicles are fit for purpose and are equipped with suitable fastenings and load points. Operators must be equipped with steel toe-capped footwear to ISO20345:2011, and gloves if appropriate. There must be a sufficient number of operators, both on and off the vehicle to handle items safely. If the loading activity is taking place on or near a public thoroughfare (even if on private land), operators must wear high-visibility clothing. If the loading activity is taking place on or near a public thoroughfare (even if on private land), then barriers to demarcate the hazardous area should be erected. As items to be unloaded will have to be handled, all moving and lifting operations must refer to a Manual Handling risk assessment and safe system of work (such as T.I.L.E.). It is recommended that a tail-lift or ramp is used to move loads onto the vehicle, especially if the loads have wheels or castors, for example, flight cases or meat-racks. | 4 | 2 | 8 |  |  |
| Directing vehicles | Crush injuries | Operators, persons nearby | 4 | 4 | 16 | If directing vehicles that are reversing, care must be taken to ensure that a line of sight is maintained at all times with the driver, usually by ensuring that the operator can see the vehicle mirrors. Exclude any members of the public from the area where the vehicle is reversing, either with barriers, or other operators whose sole responsibility is preventing members of the public straying into the path of the vehicle. All operators involved in directing vehicles must wear high –visibility clothing. | 4 | 1 | 4 |  |  |

| Risk Scoring Key | Severity |  | Likelihood |  |
| --- | --- | --- | --- | --- |
|  | Fatality | 5 | Certain or imminent | 5 |
|  | Major injury, disabling illness, major damage | 4 | Very likely | 4 |
|  | Lost time injury, illness, damage | 3 | May happen | 3 |
|  | Minor injury, minor damage | 2 | Unlikely | 2 |
|  | Delay only | 1 | Very unlikely | 1 |

| Risk Rating Categories | 10 to 25 | Unacceptable | Do not proceed; seek immediate guidance from the safety team |
| --- | --- | --- | --- |
|  | 6 to 9 | Tolerable | Proceed with caution but seek to reduce risk further if possible |
|  | 1 to 5 | Acceptable | Proceed |

| Order of Control Measures | Most effective | Eliminate | Ask yourself if the activity needs to be carried out |
| --- | --- | --- | --- |
|  |  | Substitute | Ask yourself if the same effect can be achieved with something less risky |
|  |  | Reduce | Ask yourself if you can use less of something, or limit the time etc |
|  |  | Isolate | Make sure that the risk is contained to the smallest possible area |
|  |  | Enclose | Make sure that no-one can get to the hazard |
|  |  | Other Engineering Controls | Emergency stop buttons, automated controls etc |
|  |  | Safe System of Work | Carry out the work according to a specific step by step programme with training |
|  |  | Training/Communication | Safety team can advise |
|  |  | PPE | Use of ear defenders, hard hat, toetectors etc |
|  | Least effective | Discipline and Enforcement | Telling people to be careful |

| Assessment date: 08/05/2018 | Assessor: Jonathan Hill |
| --- | --- |
| Review date: 21/02/19 | Reviewer: Jonathan Hill |
| Review History: 14/07/18: Added control measures regarding ensuring vehicle engine is off before activities commence 21/02/19: Language clarified |  |