Safety Risk Assessment Form

| Reference: | RBC Production Generic 011 – Use of ladders |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date: | 14/07/18 | Assessor(s): | Jonathan Hill |  |  |  | Department/Area: |  |  | Campus |  |
| Activity Describe the activity and the ways in which it could pose a risk | Hazards e.g. personal injury, electric shock, people or objects falling from height, crushing | Persons at risk | Risk before control measures |  |  | Controls Describe controls used to reduce risk to a tolerable or preferably acceptable rating | Residual Risk after control measures |  |  | Additional controls? e.g. monitor and review, suggestions for future activities | Owner Person responsible for the controls |
|  |  |  | Severity | Likeli-hood | SxL |  | Severity | Likeli-hood | SxL |  |  |
| Use of ladders | Ladder collapsing | Operator, persons nearby | 4 | 3 | 12 | Carry out a visual inspection of the ladder before any use, checking for any damage or missing elements. Damage to, or parts missing must be reported to a member of the Technical Department immediately. Erect the ladder as per the manufacturers instructions, ensuring that any platforms, locking pins or bars and stabilisers are used and deployed correctly. The ladder must only be erected on firm, flat ground. The ladder should be footed when in use. In the case of Zarges Skymaster Industrial or Trade combination ladders, used in A-frame configuration, the footer must not stand on the stabiliser bar. Never exceed the Safe Working Load (SWL) of the ladder. All ladders used in venues should conform to EN131 which mandates a SWL of 150kg. Do not overreach whilst on the ladder. Move the ladder if necessary. Maintain three points of contact with the ladder at all times when stationary. Ladders must not be used by persons under the influence of alcohol or drugs, including some prescription drugs. | 2 | 3 | 6 |  | Operator, students, Technicians, other staff |
|  | Objects falling | Persons nearby | 4 | 3 | 12 | The operator must empty their pockets before ascending. Tools must be lanyarded back to a suitable fixing. Heavy items are not to be carried up the ladder, use a line and pulley to take the weight. Any persons footing the ladder must wear a hard hat to EN397. | 1 | 3 | 3 |  | Operator, footer, students, Technicians, other staff |
|  | Contact with overhead structures | Operators | 3 | 4 | 12 | Before ascending the ladder, ensure that you are aware of any overhead obstructions, such as air handling ducts, scaffold clamps, overhead bars. If your ascent is near any overhead obstructions, the use of a hard hat to minimum EN397, equipped with a chin strap is recommended. | 3 | 2 | 6 |  | Operators |

| Risk Scoring Key | Severity |  | Likelihood |  |
| --- | --- | --- | --- | --- |
|  | Fatality | 5 | Certain or imminent | 5 |
|  | Major injury, disabling illness, major damage | 4 | Very likely | 4 |
|  | Lost time injury, illness, damage | 3 | May happen | 3 |
|  | Minor injury, minor damage | 2 | Unlikely | 2 |
|  | Delay only | 1 | Very unlikely | 1 |

| Risk Rating Categories | 10 to 25 | Unacceptable | Do not proceed; seek immediate guidance from the safety team |
| --- | --- | --- | --- |
|  | 6 to 9 | Tolerable | Proceed with caution but seek to reduce risk further if possible |
|  | 1 to 5 | Acceptable | Proceed |

| Order of Control Measures | Most effective | Eliminate | Ask yourself if the activity needs to be carried out |
| --- | --- | --- | --- |
|  |  | Substitute | Ask yourself if the same effect can be achieved with something less risky |
|  |  | Reduce | Ask yourself if you can use less of something, or limit the time etc |
|  |  | Isolate | Make sure that the risk is contained to the smallest possible area |
|  |  | Enclose | Make sure that no-one can get to the hazard |
|  |  | Other Engineering Controls | Emergency stop buttons, automated controls etc |
|  |  | Safe System of Work | Carry out the work according to a specific step by step programme with training |
|  |  | Training/Communication | Safety team can advise |
|  |  | PPE | Use of ear defenders, hard hat, toetectors etc |
|  | Least effective | Discipline and Enforcement | Telling people to be careful |

| Assessment date: 14/07/18 | Assessor: Jonathan Hill |
| --- | --- |
| Review date: 29/11/19 | Reviewer: Jonathan Hill |
| Review History: 29/11/19 Added overhead clearance check |  |